Contact:									Date: _____________________

_________________________________________

(Your company name)

_________________________________________

(Your contact person)

_________________________________________

(Your phone number and e-mail)

## FOR IMMEDIATE RELEASE

CAMBRIDGE OFFERS A DO-IT-YOURSELF ENGINEERED PAVEMENT SYSTEM

FOR YOUR DRIVEWAY, POOL DECK, PATIO OR SUNROOM

While many homeowners can appreciate the old world beauty of an interlocking pavingstone system instead of traditional concrete or asphalt, what sometimes goes unknown is that pavingstones are stronger than poured concrete, easier to install and virtually maintenance-free.

Cambridge Pavingstones™, made by Cambridge Pavers, Inc. in Lyndhurst, New Jersey, for example, can be installed directly over existing concrete or asphalt.  A Cambridge Interlocking Pavement System is mortarless and set-in-sand so installation, removal and replacement of individual stones is easy.

If you're considering a weekend do-it-yourself pavement project, here's the equipment you'll need (what you don't have you can easily rent): one pair each of gloves and safety glas

ses; carpenter's chalk line; stakes; spade; stone cutter or masonry saw; plate compactor; 8-10', straight 2"x4" yellow pine; (2) 8' lengths of 1 1/2" O.D. PVC pipe; nylon mason string; tape measure; broom; and line level.

The following are abbreviated "How To" instructions for installing your own patio, walkway or any application where you're considering a flexible, interlocking pavement system.  For a free DesignScaping Idea Book with dozens of pattern ideas, call The Cambridge Center at 201.933.5000 or any Authorized Cambridge Pavingstone distributor. Distributors locations are also easy to find on the Cambridge Web site (www.cambridgepavers.com)

-more-

Cambridge Do-It-Yourself Page 2

STEP 1- PREPARATION

•  Sketch a diagram of the area to be paved.

•  Square, rectangular, circular and free-forming projects are all possible.

•  To guide pavingstone lines, it is best to square off a 90° corner.  If there is not an existing structure to measure off of, use a "3-4-5 triangle" method. Begin by measuring 3' along a straight

reference.  Measure 4' from one end of the 3' line, drive a stake where 4' and 5' marks meet.  The angle opposite the 5' side will be 90°.  Once this corner is done extend the other sides using additional "3-4-5 triangles".

•  Set stakes, for outside perimeters, 6" away from area to be paved.

•  Set string lines to establish finished patio height and slope.

•  Level the strings around the project using a string line level.

Adjust sides for water runoff by marking stakes below string line to allow for a 1" drop every 8'.  Re-adjust the string lines to follow slope marks.  The string line represents the surface of the completed job.

STEP 2-EXCAVATION

•  Skim last 1/2" off with spade to leave soil undisturbed.  Use chart to determine depth and gravel/sand requirements:

STEP 3-BASE

Fill in the excavated area with the quarry process gravel flattening, compacting and sloping.  Use the 2 x 4 and cross string lines as a guide to assure a perfect base installation.  Proper compaction and a smooth pitched surface base are vital.

-more-

# Cambridge Do-It-Yourself  Page 3

STEP 4-EDGING

•  Install a curb or edge restraint to prevent spreading of the pavers.

•  To determine where you want to install your edge restraint, lay a row of pavers with manufactured edgers, when available, from one side of the area to be paved to the other.  Position your curb so that you maximize the use of whole pavers and minimize the number of stones you need to cut. Measure this distance and transfer the measurements to the curb area.  Position your Pave Edge restraint and nail it into the aggregate base.

STEP 5-LEVELING YOUR SAND

•   Add the coarse sand and loosely level it to 3/4’ - 1" using the PVC pipes as guides or rails and the 2 x 4 board.  Do approximately 8' sections and be sure to lay pavers before leveling the next section.  Use a trowel to fill in the gaps left behind by the pipes after you remove them.  Do not walk on or disturb the freshly leveled sand.

STEP 6-LAYING THE PAVERS

•  Lay the pavers onto the leveled sand, beginning at the right angle.  There are many shape and pattern possibilities.  DesignScaping pattern sheets are available from Authorized Cambridge distributors and online (www.cambridgepavers.com).

•  Each Cambridge Pavingstone is manufactured with a spacer bar to assure constant joint widths, this allows the sand to fill the joints.

•   Just hold a paver so the bottom 1/4" to 1/2" should click against the top edge of the paver already in the sand.  Release your grasp dropping the paver directly downward.

•   Use additional string lines to insure straight paver joints.

•   Check that the rows of pavers are straight every 3' by running a string line along the front of the rows - making sure each paver touches the string.

•   Always install pavers working off multiple cubes if project allows. Remove pavers from the cubes by bands vertically.

- more -

# Cambridge Do-It-Yourself Page 6

STEP 7-CUTTING

•  Once you have laid all the pavers that will fit your area, you may need to cut some to fit.

•  Use your stone cutter for simple cross cuts or a power driven masonry saw for angled cuts.

STEP 8- COMPACTING THE PAVERS

•  Using the plate compactor, make two passes over the installed pavers.  This will seat the paver, forcing sand up between the joint and completing the interlocking effect of the Cambridge Pavingstone System.

STEP 9-FINAL SWEEPING AND COMPACTION

•  Sweep more sand over the surface until all joints are filled to the bottom of the chamfered edge.

•  Compact again.

•  Repeat process until joints are filled.

-END-